Cavli C16QS PPP Setup on Raspberry Pi

External Airtel SIM + PPPD + ppp0 + google.com ping

# 1. Goal

This document explains how to configure a Cavli C16QS development board on a Raspberry Pi, start PPP mode, create the ppp0 network interface, and verify internet connectivity by pinging google.com.

# 2. Final working architecture

Raspberry Pi USB port
        ↓
Cavli C16QS development board
        ↓
External Airtel SIM
        ↓
Airtel LTE network
        ↓
ppp0 network interface on Raspberry Pi
        ↓
ping -I ppp0 google.com

# 3. Known working values

| Item | Value |
| --- | --- |
| Modem/module | Cavli C16QS development board |
| SIM used | External Airtel India SIM |
| APN | airtelgprs.com |
| USB serial ports on Raspberry Pi | /dev/ttyACM0, /dev/ttyACM1, /dev/ttyACM2 |
| Working AT/PPP port in this test | /dev/ttyACM0 |
| Baud rate | 115200 |
| PPP start command | AT+PPPSTART |
| Successful PPP response | CONNECT |
| PPP interface created by Linux | ppp0 |
| Successful final test | ping -I ppp0 google.com |

# 4. Important rule before starting

After AT+PPPSTART returns CONNECT, the serial port is no longer in AT-command mode. It becomes PPP binary data mode. Closing minicom does not return it to AT mode. If you manually test AT+PPPSTART, reset or power-cycle the Cavli board before running pppd.

# 5. Install required packages

Run these commands on Raspberry Pi:

sudo apt update
sudo apt install ppp minicom python3-serial iputils-ping curl

# 6. Check that the Cavli board is detected

lsusb
ls /dev/ttyACM*
dmesg | grep -i acm

Expected serial devices:

/dev/ttyACM0  /dev/ttyACM1  /dev/ttyACM2

# 7. One-time external SIM verification

Use minicom only for the initial SIM check. Do not send AT+PPPSTART in minicom unless you plan to reset the board afterward.

sudo minicom -D /dev/ttyACM0 -b 115200

Type these commands:

AT
ATE0
AT+CMEE=2
AT^SIMSWAP=1
AT+CFUN=0
AT+CFUN=1

Wait around 15 to 30 seconds, then type:

AT+CPIN?
AT+CIMI
AT+ICCID
AT+CSQ
AT+COPS?
AT+CEREG?
AT+CGDCONT=1,"IP","airtelgprs.com"
AT+CGATT?

Good expected results include:

+CPIN: READY
+CEREG: 0,1    or    +CEREG: 0,5
+CGATT: 1

Exit minicom with Ctrl+A, then X. If you accidentally send AT+PPPSTART and get CONNECT, reset or power-cycle the Cavli board before continuing.

# 8. Create the pppd connect script

Save the following program in this exact location:

/usr/local/bin/cavli-connect.py

Create it with:

sudo nano /usr/local/bin/cavli-connect.py

Paste this program:

#!/usr/bin/env python3

"""
Cavli C16QS pppd connect script for Raspberry Pi.

pppd opens /dev/ttyACM0 first. Then pppd runs this script and connects
this script's stdin/stdout to the modem serial port.

This script sends AT commands and exits only after AT+PPPSTART returns CONNECT.
After this script exits with code 0, pppd immediately starts PPP negotiation
on the same open serial port.
"""

import os
import sys
import time
import select

APN = "airtelgprs.com"


def log(msg):
    print(msg, file=sys.stderr, flush=True)


def modem_write(text):
    os.write(sys.stdout.fileno(), (text + "\r").encode())
    sys.stdout.flush()
    log(f">>> {text}")


def modem_read(timeout=5):
    end_time = time.time() + timeout
    response = b""

    while time.time() < end_time:
        remaining = max(0.1, end_time - time.time())
        readable, _, _ = select.select([sys.stdin], [], [], remaining)

        if readable:
            chunk = os.read(sys.stdin.fileno(), 1024)
            if chunk:
                response += chunk
            else:
                break

    text = response.decode(errors="ignore")
    if text.strip():
        log(text.strip())
    return text


def send_at(command, ok_list, timeout=8, retries=1):
    for attempt in range(retries + 1):
        modem_write(command)
        response = modem_read(timeout)

        for ok in ok_list:
            if ok in response:
                return True, response

        if "ERROR" in response or "+CME ERROR" in response or "+CMS ERROR" in response:
            log(f"Command returned ERROR: {command}")
            return False, response

        log(f"Timeout waiting for {ok_list} after command: {command}")

        if attempt < retries:
            time.sleep(1)

    return False, response


def main():
    log("Cavli connect script started")

    ok, _ = send_at("AT", ["OK"], timeout=5, retries=2)
    if not ok:
        log("Failed at AT")
        return 1

    ok, _ = send_at("ATE0", ["OK"], timeout=5, retries=1)
    if not ok:
        log("Failed at ATE0")
        return 1

    # Enable verbose modem errors.
    send_at("AT+CMEE=2", ["OK"], timeout=5, retries=0)

    # Ensure Airtel APN is configured.
    ok, _ = send_at(f'AT+CGDCONT=1,"IP","{APN}"', ["OK"], timeout=8, retries=1)
    if not ok:
        log("Failed at AT+CGDCONT")
        return 1

    # Optional packet attach check. Continue if it only prints OK.
    send_at("AT+CGATT?", ["OK"], timeout=8, retries=0)

    # Start Cavli PPP data mode.
    ok, _ = send_at("AT+PPPSTART", ["CONNECT", "+PPPSTART"], timeout=15, retries=1)
    if ok:
        log("PPPSTART successful. CONNECT received.")
        log("Exiting connect script so pppd can start PPP negotiation.")
        return 0

    # Optional fallback for firmware that supports standard dial PPP.
    log("AT+PPPSTART failed. Trying fallback ATD*99# ...")
    ok, _ = send_at("ATD*99#", ["CONNECT"], timeout=15, retries=0)
    if ok:
        log("Fallback dial successful. CONNECT received.")
        return 0

    log("No PPP CONNECT response.")
    return 1


if __name__ == "__main__":
    sys.exit(main())

Make it executable:

sudo chmod +x /usr/local/bin/cavli-connect.py

# 9. Create the pppd peer file

Save the PPP configuration file in this exact location:

/etc/ppp/peers/cavli

Create it with:

sudo nano /etc/ppp/peers/cavli

Paste this configuration:

/dev/ttyACM0
115200

connect "/usr/local/bin/cavli-connect.py"

noauth
defaultroute
replacedefaultroute
usepeerdns

local
nocrtscts
lock
debug
nodetach

asyncmap 0
default-asyncmap

novj
novjccomp
nopcomp
noaccomp

ipcp-accept-local
ipcp-accept-remote

lcp-echo-interval 30
lcp-echo-failure 4

If your working AT/PPP port is not /dev/ttyACM0, change the first line to /dev/ttyACM1 or /dev/ttyACM2.

# 10. Start PPP

Before starting, reset or power-cycle the Cavli board. Then run:

sudo pppd call cavli

Good output includes:

Cavli connect script started
>>> AT
OK
>>> AT+PPPSTART
CONNECT
PPPSTART successful. CONNECT received.
Using interface ppp0
Connect: ppp0 <--> /dev/ttyACM0
local  IP address ...
remote IP address ...

# 11. Ping google.com through ppp0

Open a second terminal while pppd is still running. Check the PPP interface:

ip addr show ppp0

First test raw IP connectivity:

ping -I ppp0 8.8.8.8

Then test DNS and internet routing:

ping -I ppp0 google.com

If this command works, PPP is fully working on Raspberry Pi.

# 12. Other useful tests

ip route
cat /etc/ppp/resolv.conf
curl --interface ppp0 https://api.ipify.org
curl --interface ppp0 -I https://google.com

# 13. Stop PPP

In the terminal where pppd is running, press Ctrl+C. Or run:

sudo killall pppd

# 14. Troubleshooting

| Symptom | Fix |
| --- | --- |
| No /dev/ttyACM* ports | Check USB cable, power, and lsusb. The modem may appear as Qualcomm 05c6:9330. |
| AT works in minicom but pppd fails | Reset the Cavli board and do not manually send AT+PPPSTART before running pppd. |
| Script shows repeated LCP ConfReq | Try another port: change /dev/ttyACM0 to /dev/ttyACM1 or /dev/ttyACM2 in /etc/ppp/peers/cavli. |
| ping -I ppp0 8.8.8.8 works but google.com fails | DNS issue. Check /etc/ppp/resolv.conf or temporarily set nameserver 8.8.8.8. |
| AT+PPPSTART gives CONNECT but no ping | Make sure pppd starts PPP immediately after CONNECT using the Python connect script. |
| Need AT mode again after CONNECT | Stop pppd. If AT still does not respond, reset or power-cycle the Cavli board. |

# 15. Final success checklist

Cavli board visible as /dev/ttyACM0, /dev/ttyACM1, /dev/ttyACM2.

External Airtel SIM returns +CPIN: READY.

APN is airtelgprs.com.

AT+PPPSTART returns CONNECT.

pppd creates ppp0.

ping -I ppp0 8.8.8.8 works.

ping -I ppp0 google.com works.